SECTION 08620

LOW PROFILE MODEL (LPG and LPG-S) SKYLIGHTS

THERMALIZED

PART 1-GENERAL

  - SUMMARY

- This section includes the following:

  - LPG model deck-mounted glass glazed skylights.

  - LPGS model curb-mounted glass glazed skylights.

- Related Sections:  The following sections contain requirements that relate to this section:

  - Division Six Section “Rough Carpentry” for wood curbs and nailers.

  - Division Seven Section “Roof Accessories” for curbs, roof hatches, and smoke vents.

  - Division Seven Section “Flashing and Sheet Metal” for metal flashing for skylights.

  - Division Eight Section “Metal-Framed Skylights” for large site constructed skylights.

C.  Refer to roofing system sections for roofing accessories to be built into the roofing system to accommodate work of this section.

  - PERFORMANCE REQUIREMENTS

- General:  Provide glass glazed skylights capable of withstanding loads as defined by the local codes having jurisdiction where units are to be installed without failure.

- Units shall be tested to compliance with AAMA\WDMA\CSA\101\I.S.2\A440.

- Units shall be tested to a Performance Grade of PG100 1981 x 1981 minimum per AAMA\WDMA\CSA\101\I.S.2\A440.

- Units shall provide no water penetration per ASTM E 331 when tested at 15.04 psf.

1.3   SUBMITTALS

- Product Data Sheet:  For each type of skylight specified, include details of construction and installation, relative to applicable curb configuration.

- Samples for Selection:  Manufacturer’s color charts showing a full range of colors available for each type of skylight glazing and Aluminum Finish.

  - GLAZING MATERIALS

- Insulating Glass: 1 1/8 inch thick consisting of ¼ inch tempered exterior lite, ½ inch sealed air space and 7/16 inch laminated safety glass interior lite. (as specified in section 08800)

- Glazing Gaskets: Manufacturer’s proprietary pressure-glazing gaskets of elastomer type and hardness selected by the skylight manufacturer to comply with requirements. Glazing gaskets to be extruded thermoplastic elastomer by the skylight manufacturer.

- Spacers, Edge Blocks and Setting Blocks: Manufacturer’s standard permanent non-migrating type of elastomer type and hardness selected to comply with requirements. Spacers, Edge Blocks, and Setting Blocks to be extruded thermoplastic elastomer by the skylight manufacturer.

- Glazing Weatherseal Sealant: Neutral-curing silicone sealant recommended by skylight and sealant manufacturers for this use and furnished by the skylight manufacturer.

    - Sealant is capable of withstanding 50 percent movement in both extension and compression (total of 100 percent movement) when tested for adhesion and cohesion under maximum cyclic movement according to ASTM C 719.

    - Sealant complies with ASTM C 920 for Type S, Grade NS, Class 25, Uses NT, G, A, and O, as applicable to substrates including other sealants with which it comes in contact.

    - Color: Black

  - WARRANTY

- General:  Warranties specified in this section shall not deprive the Owner of other rights the Owner may have under other provisions of the Contract Documents and shall be in addition to and run concurrent with other warranties made by the Contractor under requirements of the Contract Documents.

- Skylight Warranty:  Provide written warranty signed by manufacturer, agreeing to repair or replace work that exhibits defects in materials or workmanship and guaranteeing weather-tight and leak-free performance.  “Defects” is defined as uncontrolled leakage of water and abnormal aging or deterioration.

  - Warranty Period:  2 years from date of Substantial Completion.

- Finish Warranty:  Provide written warranty signed by manufacturer agreeing to repair or replace work with finish defects.  “Defects” is defined as peeling, chipping, chalking, fading, abnormal aging or deterioration, and failure to perform as required.

  - Warranty Period for Anodized Finish:  1 year from date of Substantial Completion.

  - Warranty Period for Powder Coat Finish:  5 years from date of Substantial Completion.

  - Warranty Period for Kynar 500 Finish:  5 years from date of Substantial Completion. (10 and 20 years available if specified)

PART 2- PRODUCTS

  - MANUFACTURERS

- Manufacturers:  Subject to compliance with requirements, provide products by Wasco Skylights part of the Velux Group, Wells, ME (800-388-0293).

- Substitutions:  Manufactures shall not be considered without prior approval in writing no later than ten (10) calendar days prior to bid.  Substitute manufacturers must have been in the custom skylight business for not less than a period of 15 years and must submit to the Architect the following:

  - List of similar projects successfully completed within the last five years.

  - Proof of financial capability.

  - Complete details of proposed skylight.

  - Complete specifications for Architect’s review.

  - FRAMING MATERIALS

- Curb Frame:  Extruded aluminum alloy 6063-T5 (min.) ASTM B 221 (ASTM B 221 M) with minimum effective thickness of .094 inches.  Curb frame to have glass fiber reinforced polymer thermal break.

  - Poured and de-bridged polyurethane thermal breaks are not acceptable.

- Retainer Frame:  Extruded aluminum alloy 6063-T5 (min).  ASTM B 221 (ASTM B 221 M) with minimum effective thickness of 0.60 inches.

- Support Structure Deck Mounted: Minimum 1 ½” wide below decking material at perimeter and under all rafters and purlins. Purlins can be self supporting consult manufacturer for limitations.

- Support Structure Curb Mounted: Minimum 1 ½” wide at perimeter and under all rafters and purlins. Purlins can be self supporting consult manufacturer for limitations.

  - Slope:  Support structure must slope a minimum of 3:12 pitch.

- Glazing: Insulated glass; 1 1/8 inch thick consisting of ¼ inch tempered exterior lite, ½ inch sealed air space and 7/16 inch laminated safety glass interior lite. (as specified in section 08800).

- Fasteners:  Same metal as metals being fastened, or nonmagnetic stainless steel or other non-corrosive metal as recommended by Wasco Products, Inc.

- Bituminous Coating:  SSPC-Paint 12, solvent-type, bituminous mastic, nominally free of sulfur and containing no asbestos fibers, compounded for 15-mil (0.4 mm) dry film thickness per coating.

  - SKYLIGHT UNITS

- General:  Factory-assembled or knocked down, deck mounted or curb-mounted unit consisting of insulated glass, gasketing, inner frame designed to mount directly to the roof deck or on separate curb with self-contained flashing.

  - Products:  Provide Model LPG (deck mount) or LPGS (curb mount) meeting the requirements of this section.

- Condensation Control:  Provide integral condensation gutter system with corners fully welded for waterproof quality and weeps to collect and dispose of condensation to the outside.

- Thermal Break Frames:  Provide glass fiber reinforced polymer thermal break.

- Shape and Size:  As indicated on project drawings.

  - FABRICATION

- Framing Components:  As follows:

1.    Factory fit and assemble components.

  - Fabricate components that, when assembled, will have accurately fitted joints with ends coped or mitered to produce hairline joints free of burrs and distortion.

  - Fabricate components to drain water passing joints and to drain condensation and moisture occurring or migrating within skylight system to the exterior.

  - Fabricate components to accommodate expansion, contraction, and field adjustment, and to provide for minimum clearance and shimming at skylight perimeter.

  - Form shapes with sharp profiles, straight and free of defects or deformations, before finishing.

  - Fit and secure joints by heliarc welding.

  - ALUMINUM FINISHES (EDIT AS REQUIRED)

- General:  Comply with NAAMM “Metal Finishes Manual” recommendations for application and designations of finishes.

- Finish designations prefixed by AA conform to the system for designations of aluminum finishes established by the Aluminum Association.

  - Mill Finish:  Manufacturer’s standard mill finish.

  - Clear-Anodized Finish, Class I:  AA-C22A41 complying with AAMA 611.

  - Color –Anodized Finish Class I:  AA-C22A42/A44 complying with AAMA 611.  Color:______________

  - Kynar Fluoropolymer Two-Coat System:  (70% PVDF) complying with AAMA 2605.  Color:______________ (3coat and 4 coat available)

  - Powder Coat High-Performance Architectural Coating: comply with AAMA 2604. Color:______________

PART 3- EXECUTION

  - EXAMINATION

- Examine substrates and conditions, with installer present, for compliance with requirements for installation tolerances and other conditions affecting skylight performance.

    - Proceed with installation only after unsatisfactory conditions have been corrected.

  - PREPARATION

- Metal Protection:  As follows:

  - Where aluminum will contact dissimilar metals, protect against galvanic action by paining contact surfaces with primer or by applying sealant or tape recommended by manufacturer for this purpose.

  - Where aluminum will contact concrete or masonry, protect against corrosion by painting contact surfaces with bituminous paint.

  - Where aluminum will contact pressure-treated wood, separate dissimilar materials by methods recommended by manufacturer.

  - INSTALLATION

- General:  Comply with manufacturer’s written instructions for protecting, handling, and installing skylight components.

- Coordinate with installation of roof deck and other substrates to receive skylight units.

- Coordinate with installation of vapor barriers, roof insulation, roofing, and flashing as required to assure that each element of the work performs properly and that combined elements are waterproof and weather tight.  Anchor units securely to supporting structural substrates, adequate to withstand lateral and thermal stresses ass well as inward and outward loading pressures.

- Counter Flashing:  Where counter flashing is required as component of the skylight, install to provide an adequate waterproof overlap with roofing or roof flashing (as counter flashing).  Seal with thick bead of mastic sealant, except where overlap is indicated to be left open for ventilation.

  - CLEANING AND PROTECTION

- Clean exposed metal and plastic surfaces according to manufacturer’s instructions during installation.  Touch up damaged metal coatings.

- Final cleaning by others.

END OF SECTION